Ohm’s law and combinations of resistors

The voltage (or potential difference) is a push that charges experience.  The potential difference is measured in volts, a unit named after Alessandro Volta.  This force may result in charges moving – otherwise known as a current.  The current is measured in amperes or amps, a unit named after André-Marie Ampère.  Actually an ampere is a fairly large current and we will see currents measured in mA (milliamps).  There is a class of materials and devices made from those materials in which the current in the material is directly proportional to the voltage applied to the material; so that if one doubled the voltage, one would double the resulting current; if one tripled the voltage, one would triple the resulting current; and so on.  This relationship is expressed as

V = I R

and is called Ohm’s law.  Furthermore, the proportionality constant R is called the resistance; resistance is measured in ohms, a unit named after Georg Simon Ohm. Actually an ohm is a fairly small resistance and we will see resistances measured in k (kilo-ohms).

| ### Resistor |  |
| --- | --- |
| Voltage (V) | Current (mA) |
| 1.0 | 0.29 |
| 2.0 | 0.61 |
| 3.0 | 0.88 |
| 4.0 | 1.19 |
| 5.0 | 1.53 |

Make a plot of current versus voltage.  Add a fit to a straight line. Force that line to go through the origin. Paste the plot below.

Paste plot here.

Since the data has current on the Y axis, solve Ohm’s law for current I. Type the result below.

Ohm’s Law solved for I

According to the above result what is the interpretation of the slope of your fit equation?

Slope Interpretation:

Finally determine the resistance from your graph (don’t forget the unit).

Resistance: