1. Suppose you have the 32-bit integer written in Hex as 38C0FB3D.  How would it be stored in memory in a system using the big-endian (byte order) approach?

| Big-endian |  |
| --- | --- |
| Memory Location | Value |
| 500 | 38 |
| 501 | C0 |
| 502 | FB |
| 503 | 3D |
| 504 |  |
| 505 |  |
| 506 |  |
| 507 |  |
| 508 |  |

2. Interpret the following eight-bit binary string

| 1 | 1 | 1 | 0 | 1 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| A. As an unsigned integer | 232 |
| --- | --- |

11101000

00010111

+1 =

00011000

24

| B. As a signed integer | -24 |
| --- | --- |

| C. As a hexadecimal number |  |
| --- | --- |

3. Construct the 16's complement of the decimal number 743.

|  | 1 | 1 |  |
| --- | --- | --- | --- |
|  | 7 | 4 | 3 |
| + | 8 | B | D |
|  | 0 | 0 | 0 |

4.  Show the state of the stack at each stage of the following postfix calculation.  Use a column of the table to represent what’s on the stack at that stage.  Start at the left and proceed to the right.

8 7 6 5 * + 3 4 * + 1 + *

|  |  |  | 5 |  |  |  | 4 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 6 | 6 | 30 |  | 3 | 3 | 12 |  | 1 |  |  |  |  |
|  | 7 | 7 | 7 | 7 | 37 | 37 | 37 | 37 | 49 | 49 | 50 |  |  |  |
| 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 400 |  |  |

Time 

5. Assuming a single-bit error and even parity, locate the offending bit from the following information.

| 0001 | 0010 | 0011 | 0100 | 0101 | 0110 | 0111 | 1000 | 1001 | 1010 | 1011 | 1100 | 1101 | 1110 | 1111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 1 |

| # group | # violating parity yes or no | # Position of the offending bit |
| --- | --- | --- |
| 1 | 1 (yes) | 1001 = 9 |
| 2 | 0 (no) |  |
| 4 | 0 (no) |  |
| 8 | 1 (yes) |  |

6. Fill in the table below with the corresponding infix.

| Infix | Postfix |
| --- | --- |
|  | 8 3 5 + * 7 2 6 * + 4 * + |

8 3 5 + * 7 2 6 * + 4 * +

8 (3 + 5) * 7 2 6 * + 4 * +

8 * (3 + 5) 7 2 6 * + 4 * +

8 * (3 + 5) 7 2 * 6 + 4 * +

8 * (3 + 5) (7 + 2 * 6) 4 * +

8 * (3 + 5) (7 + 2 * 6) * 4  +

8 * (3 + 5) + (7 + 2 * 6) * 4

7. Enter the binary string into the CRC generator.

| 0 |  | 0 | 0 | 0 |  |  |  | 10110001101 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| The first four just shift in |  |  |  |  |  |  |  |  |
| 1 |  | 0 | 1 | 1 |  |  | 0001101 |  |
| 1 |  | 1 | 1 | 1 |  |  | 001101 |  |
| 0 |  | 1 | 1 | 1 |  |  | 01101 |  |
| 1 |  | 1 | 1 | 0 |  |  |  | 1101 |

8. Express -9085.789 as a float.  Use 1 bit for the sign, 23 bits for the mantissa, 8 bits for the exponent.  Use a bias of 128 for the exponent. Display the leading 1. (In other words don’t use a website.)

Sign

1

9085  10 0011 0111 1101 (14 places  9 places to go)

.789  .789 * 2^9 = 403.968  403  1 1001 0011

Mantissa

| 1 | 0 | 0 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1. | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

14 14+128 = 142  1000 1110

Exponent

| 1 | 0 | 0 | 0 | 1 | 1 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

9. The following program and data are stored in memory.  Determine the value in the accumulator just before executing each line.

| Address | Memory Value | Value in Accumulator A before evaluating line of code |
| --- | --- | --- |
| 0 | LOAD IMMEDIATE 6 | XXX |
| 1 | ADD INDIRECT 5 | 6 |
| 2 | ADD 8 | 6+6=12 |
| 3 | STOP | 12+6 = 18 |
| 4 | 9 |  |
| 5 | 8 |  |
| 6 | 10 |  |
| 7 | 5 |  |
| 8 | 6 |  |
| 9 | 4 |  |
| 10 | 7 |  |
| 11 | 12 |  |
| 12 | 11 |  |

10. Express the following float as a decimal number. Show any intermediate steps.

Sign

1

Mantissa

| 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Exponent

| 0 | 1 | 1 | 0 | 1 | 1 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Sign: negative Exponent (assume bias of 128): 01101100  108  108-128 = -20 Mantissa: 111 0011 1001 1100 1110 0111  7,576,807  7,576,807/2^23 = 0.90322577953338623046875 Decimal/Scientific Notation evaluation: - (0.90322577953338623046875)*2^(-20) -8.61383228 e^-7 -8.6138322785700438544154167175293e-7

11. Determine the Hamming-code parity bits for the following data. Assume even parity.

| 0001 | 0010 | 0011 | 0100 | 0101 | 0110 | 0111 | 1000 | 1001 | 1010 | 1011 | 1100 | 1101 | 1110 | 1111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| 1 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |

12. Fill in the table below with the corresponding postfix.

| Infix | Postfix |
| --- | --- |
| (9 * 6 + 5 * 8) + (4 + 2 * 3) |  |

(9 * 6 + 5  * 8) + (4 + 2 * 3)

(9 6 *  + 5  8 *) + (4 + 2 3 * )

9 6 * 5  8 * + + 4 2 3 * +

9 6 * 5  8 * + 4 2 3 * + +