Name:______________________

For each step in the program below, indicate whether a register is being written to by entering a “W”, read from by entering an “R” or not involved by leaving it blank.

1. ADD r2, r1, r3

2. LD r4, (r5)

3. SUM r5, r1, r4

4. MUL r3, r1, r6

5. SUB r2, r10, r12

6. ST (r12), r8

7. OR r7, r6, r12

|  | R1 | R2 | R3 | R4 | R5 | R6 | R7 | R8 | R9 | R10 | R11 | R12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |

Indicate any data dependencies and their type.

| Register | Dependency | Instructions Involved | Restriction |
| --- | --- | --- | --- |

Write the program, splitting it up for processing in two different execution units.

| Pipeline 1 | Pipeline 2 |
| --- | --- |

Write the program, splitting it up for processing by as many different execution units as possible.

| Pipeline 1 | Pipeline 2 | Pipeline 3 | Pipeline 4 |
| --- | --- | --- | --- |