| Parallel Code Exercise April 14, 2008 | Name: ___________________________ |
| --- | --- |

- Adapt the CD-DA standards to porpoises. What transmission data rate is required by your standards? Explain your reasoning.

Species Approximate Hearing Range (Hz)
Human: 64 – 23,000
Porpoise: 75 – 150,000

- For each step in the program below, indicate whether a register is being written to by entering a “W”, read from by entering an “R” or not involved by leaving it blank.

- ADD r3, r2, r1

- LD r4, (r5)

- MUL r5, r1, r9

- ST (r12), r8

- SUB r3, r4, r4

- MUL r2, r10, r12

- OR r2, r14, r12

|  | R1 | R2 | R3 | R4 | R5 | R6 | R7 | R8 | R9 | R10 | R11 | R12 | R13 | R14 | R15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Indicate any data dependencies and their type.

| Register | Dependency | Instructions Involved | Restriction |
| --- | --- | --- | --- |

Write the program, splitting it up for processing by two execution units.

Write the program, splitting it up for processing by as many different execution units so long as the total execution time is shortened.