A. Fill in the table below with the corresponding postfix.

| Infix | Postfix |
| --- | --- |
| (3 + 4 * 7 + 2) * (6 * 8 + 9) |  |

B. Assuming a single-bit error and even parity, locate the offending bit from the following information.

| 0001 | 0010 | 0011 | 0100 | 0101 | 0110 | 0111 | 1000 | 1001 | 1010 | 1011 | 1100 | 1101 | 1110 | 1111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 1 | 0 |

| # Groups violating parity | # Position of the offending bit |
| --- | --- |

C. Enter the binary string into the CRC generator.

| 0 |  | 0 | 0 | 0 |  |  |  | 10111010010 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| The first four just shift in |  |  |  |  |  |  |  |  |
| 1 |  | 0 | 1 | 1 |  |  | 1010010 |  |
|  |  |  |  |  |  |  | 010010 |  |
|  |  |  |  |  |  |  | 10010 |  |
|  |  |  |  |  |  |  |  | 0010 |

D. Express -78835.21789 as a float.  Use 1 bit for the sign, 23 bits for the mantissa, 8 bits for the exponent.  Use a bias of 128 for the exponent.

Sign

Mantissa

Exponent

E. Fill in the table below with the corresponding infix.

| Infix | Postfix |
| --- | --- |
|  | 9 2 5 * + 4 3 7 + * 6 + * |

F. Determine the Hamming-code parity bits for the following data. Assume even parity.

| 0001 | 0010 | 0011 | 0100 | 0101 | 0110 | 0111 | 1000 | 1001 | 1010 | 1011 | 1100 | 1101 | 1110 | 1111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
|  |  | 1 |  | 1 | 0 | 0 |  | 1 | 0 | 1 | 0 | 1 | 0 | 0 |

G. The following program and data are stored in memory.  Determine the value in the accumulator just before executing each line.

| Address | Memory Value | Value in Accumulator A before evaluating line of code |
| --- | --- | --- |
| 0 | LOAD INDIRECT 9 | XXX |
| 1 | ADD IMMEDIATE 10 |  |
| 2 | ADD 7 |  |
| 3 | STOP |  |
| 4 | 8 |  |
| 5 | 9 |  |
| 6 | 10 |  |
| 7 | 6 |  |
| 8 | 5 |  |
| 9 | 7 |  |
| 10 | 4 |  |
| 11 | 12 |  |
| 12 | 11 |  |

H. Express the following float as a decimal number. Show any intermediate steps.

Sign

1

Mantissa

| 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Exponent

| 0 | 0 | 1 | 0 | 1 | 1 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

I. Show the state of the stack at each stage of the following postfix calculation.  Use a column of the table to represent what’s on the stack at that stage.  Start at the left and proceed to the right.

6 3 1 + 4 + 7 5 * * 8 * +

|  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 6 | 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Time 

J. Interpret the following eight-bit binary string

| 1 | 0 | 0 | 1 | 1 | 0 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| A. As an unsigned integer |  |
| --- | --- |

| B. As a signed integer |  |
| --- | --- |

| C. As a hexadecimal number |  |
| --- | --- |

K. Construct the 16's complement of the decimal number 5B6.

|  | 5 | B | 6 |
| --- | --- | --- | --- |
| + |  |  |  |
|  | 0 | 0 | 0 |

L. Suppose you have the 32-bit integer written in Hex as 785CD04F.  How would it be stored in memory in a system using the big-endian (byte order) approach?

| Little-endian |  |
| --- | --- |
| Memory Location | Value |
| 500 |  |
| 501 |  |
| 502 |  |
| 503 |  |
| 504 |  |
| 505 |  |
| 506 |  |
| 507 |  |
| 508 |  |