| CSIT 301 | Name: _____________________________ |
| --- | --- |

More binary including stacks, postfix, etc.

- Fill in the table below with the corresponding infix or postfix expression.

| Infix | Postfix |
| --- | --- |
| ( 5 + 3 ) * (6 + 4 ) * 2 + 8 |  |
|  | 5 4 2 7 * * + 6 3 8 * + + |
| ( 1 + 5 ) * ((4 + 7) * (2 + 6 * 3)) |  |
|  | 3 4 5 8 * 6 7 2 1 + * + * * + |

- Show the state of the stack at each stage of the following postfix calculation.  Use a column of the table to represent what’s on the stack at that stage.  Start at the left and proceed to the right.

4  3  8  +  7  6  +  *  1  +  +  2 *  9 +

|  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Time 

- The following program and data are stored in memory.  Determine the value in the accumulator just before executing each line.

| Address | Value | Value in Accumulator A |
| --- | --- | --- |
| 0 | LOAD IMMEDIATE 5 | XXX |
| 1 | ADD 4 |  |
| 2 | ADD INDIRECT 6 |  |
| 3 | STOP |  |
| 4 | 7 |  |
| 5 | 9 |  |
| 6 | 8 |  |
| 7 | 6 |  |
| 8 | 5 |  |
| 9 | 4 |  |

- Determine the Hamming-code parity bits for the following two sets of data. Assume even parity.

| 0001 | 0010 | 0011 | 0100 | 0101 | 0110 | 0111 | 1000 | 1001 | 1010 | 1011 | 1100 | 1101 | 1110 | 1111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
|  |  | 1 |  | 1 | 1 | 0 |  | 0 | 1 | 1 | 1 | 0 | 1 | 0 |
|  |  | 1 |  | 0 | 1 | 1 |  | 0 | 0 | 1 | 0 | 1 | 1 | 0 |

- Assuming a single-bit error and even parity, locate the offending bit from the two following sets of information.

| 0001 | 0010 | 0011 | 0100 | 0101 | 0110 | 0111 | 1000 | 1001 | 1010 | 1011 | 1100 | 1101 | 1110 | 1111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 |
| 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |

| # Groups violating parity | # Offending bit |
| --- | --- |

- Supply the parity bit for the bytes below.

| Use odd parity |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |  |
| 1 | 0 | 0 | 1 | 0 | 1 | 1 | 0 |  |
| 1 | 0 | 1 | 1 | 0 | 1 | 0 | 1 |  |

| Use even parity |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |  |
| 0 | 0 | 1 | 0 | 1 | 1 | 1 | 0 |  |
| 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 |  |

| Use mark parity |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 |  |
| 0 | 1 | 1 | 1 | 0 | 0 | 1 | 0 |  |
| 1 | 0 | 0 | 1 | 0 | 0 | 1 | 0 |  |

| Use space parity |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 0 | 0 | 1 | 1 | 0 |  |
| 0 | 1 | 0 | 0 | 1 | 1 | 0 | 0 |  |
| 1 | 0 | 1 | 1 | 1 | 1 | 0 | 1 |  |

- Enter the binary string into the CRC generator, taking bits from the left. Note the first four bits just shift in.  Until there is a 1 in the leftmost position, it is just simple shifting.

| 0 |  | 0 | 0 | 0 |  |  |  | 10011011001 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 0 | 0 | 1 |  |  | 1011001 |  |
|  |  |  |  |  |  |  | 011001 |  |
|  |  |  |  |  |  |  | 11001 |  |
|  |  |  |  |  |  |  |  | 1001 |
|  |  |  |  |  |  |  |  | 001 |
|  |  |  |  |  |  |  |  | 01 |
|  |  |  |  |  |  |  |  | 1 |
|  |  |  |  |  |  |  |  |  |

- Transpose two neighboring bits (one should be a zero, the other a one) in the above binary string and repeat the exercise.

| 0 |  | 0 | 0 | 0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |

- Suppose you have the 32-bit integer written in Hex as 9D9BFD8D.  How would it be stored in memory in a system using the big-endian approach?

| Big-endian |  |
| --- | --- |
| Memory Location | Value |
| 500 |  |
| 501 |  |
| 502 |  |
| 503 |  |
| 504 |  |
| 505 |  |
| 506 |  |
| 507 |  |
| 508 |  |

- Suppose you have the 32-bit integer written in Hex as 775FF04D.  How would it be stored in memory in a system using the little-endian approach?

| Little-endian |  |
| --- | --- |
| Memory Location | Value |
| 500 |  |
| 501 |  |
| 502 |  |
| 503 |  |
| 504 |  |
| 505 |  |
| 506 |  |
| 507 |  |
| 508 |  |

- What are the ASCII values for ACK (special character for acknowledge) and NAK?

12a. (3 points) Express -8907.4890 as a float.  Use 1 bit for the sign, 23 bits for the mantissa, 8 bits for the exponent.  Use a bias of 128 for the exponent.

Sign

Mantissa

Exponent

12b. (3 points) Express the following float as a decimal number. Show any intermediate steps.

Sign

0

Mantissa

| 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Exponent

| 0 | 0 | 1 | 0 | 1 | 0 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |