Express 8675.309 as a float.  Use 1 bit for the sign, 23 bits for the mantissa, and 8 bits for the exponent.  Use a bias of 128 for the exponent.

Sign

Mantissa

Exponent

Express the following float as a decimal number. Show any intermediate steps.

Sign

1

Mantissa

| 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Exponent

| 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Assuming a single-bit error and even parity, locate the offending bit from the following information.

| 0001 | 0010 | 0011 | 0100 | 0101 | 0110 | 0111 | 1000 | 1001 | 1010 | 1011 | 1100 | 1101 | 1110 | 1111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| 1 | 1 | 1 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |

| # Groups violating parity | # Position of the offending bit |
| --- | --- |

If a 29-second process were broken into 6 stages, (2 requiring 4 seconds, 3 requiring 5 seconds, and 1 requiring 6 seconds), then it would take __________ (number of seconds) for an instruction to move through the pipeline as opposed to the original 29 seconds.  However, one instruction would be issuing from the pipeline every __________ (number of seconds) as opposed to 1 every 29 seconds without pipelining.

Determine the Hamming-code parity bits for the following data. Assume even parity.

| 0001 | 0010 | 0011 | 0100 | 0101 | 0110 | 0111 | 1000 | 1001 | 1010 | 1011 | 1100 | 1101 | 1110 | 1111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
|  |  | 1 |  | 1 | 0 | 1 |  | 0 | 0 | 0 | 1 | 1 | 0 | 1 |

Interpret the following eight-bit binary string

| 1 | 0 | 1 | 1 | 0 | 1 | 0 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| A. As an signed integer |  |
| --- | --- |

| B. As an unsigned integer |  |
| --- | --- |

11. (3 points) Enter the binary string into the CRC generator.

| 0 |  | 0 | 0 | 0 |  |  | 10010010110 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| The first four just shift in |  |  |  |  |  |  |  |
| 1 |  | 0 | 0 | 1 |  |  | 0010110 |
|  |  |  |  |  |  |  | 010110 |
|  |  |  |  |  |  |  | 10110 |
|  |  |  |  |  |  |  | 0110 |
|  |  |  |  |  |  |  | 110 |
|  |  |  |  |  |  |  | 10 |
|  |  |  |  |  |  |  | 0 |
|  |  |  |  |  |  |  |  |

A 2.4-GHz processor would have a period of

- 0.417 microseconds

- 2.4 microseconds

- 2.4 nanoseconds

- 0.417 nanoseconds

- none of the above

Fill in the table below with the corresponding postfix expression.

| Infix | Postfix |
| --- | --- |
| 4 * 9 + (6 + 2) + (5 + 8) * (3 + 7) + 1 |  |

Fill in the table below with the corresponding infix expression.

| Infix | Postfix |
| --- | --- |
|  | 3 2 7 * + 6 8 2 * 5 * 9 * + + |

The following program and data are stored in memory.  Determine the value in the accumulator just before executing each line.

| Address | Memory Value | Value in Accumulator A before evaluating line of code |
| --- | --- | --- |
| 0 | LOAD INDIRECT 7 | XXX |
| 1 | ADD 6 |  |
| 2 | ADD IMMEDIATE 8 |  |
| 3 | STOP |  |
| 4 | 8 |  |
| 5 | 6 |  |
| 6 | 4 |  |
| 7 | 10 |  |
| 8 | 7 |  |
| 9 | 5 |  |
| 10 | 9 |  |

For each step in the program below, indicate whether a register is being written to by entering a “W”, read from by entering an “R” or not involved by leaving it blank.

| 1. | XNOR r1, r4, r2 |
| --- | --- |
| 2. | LD r9, (r5) |
| 3. | ADD r2, r7, r6 |
| 4. | ST (r8), r12 |
| 5. | AND r3, r5, r4 |
| 6. | SUB r7, r4, r1 |
| 7. | MOD r9, r11, r2 |

|  | R1 | R2 | R3 | R4 | R5 | R6 | R7 | R8 | R9 | R10 | R11 | R12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |

Indicate any data dependencies and their type.

| Register | Dependency | Instructions Involved | Restriction |
| --- | --- | --- | --- |

Write the program, splitting it up for processing by as many different execution units as possible.

| Pipeline 1 | Pipeline 2 | Pipeline 3 | Pipeline 4 |
| --- | --- | --- | --- |