| CSIT 301 Review | Name: __________________________ |
| --- | --- |

Fill in the table below with the corresponding postfix.

| Infix | Postfix |
| --- | --- |
| ((4 + 5)* ( 3 + 1) + 8) * (7 + 2 * 6) |  |

Express -1776.1492 as a float.  Use 1 bit for the sign, 23 bits for the mantissa, 8 bits for the exponent.  Use a bias of 128 for the exponent.

Sign

Mantissa

Exponent

Assuming a single-bit error and even parity, locate the offending bit from the following information.

| 0001 | 0010 | 0011 | 0100 | 0101 | 0110 | 0111 | 1000 | 1001 | 1010 | 1011 | 1100 | 1101 | 1110 | 1111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 1 |

| # Groups violating parity | # Position of the offending bit |
| --- | --- |

Construct the 16's complement of the decimal number 4B6.

|  | 4 | B | 6 |
| --- | --- | --- | --- |
| + |  |  |  |
|  | 0 | 0 | 0 |

Enter the binary string into the CRC generator.

| 0 |  | 0 | 0 | 0 |  |  |  | 11001010101  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| The first four just shift in |  |  |  |  |  |  |  |  |
| 1 |  | 1 | 0 | 0 |  |  | 1010101 |  |
|  |  |  |  |  |  |  | 010101 |  |
|  |  |  |  |  |  |  | 10101 |  |
|  |  |  |  |  |  |  |  | 0101 |
|  |  |  |  |  |  |  |  | 101 |
|  |  |  |  |  |  |  |  | 01 |
|  |  |  |  |  |  |  |  | 1 |
|  |  |  |  |  |  |  |  |  |

What is the largest integer (signed) number that can be represented using 14 bits?

The following program and data are stored in memory.  Determine the value in the accumulator just before executing each line.

| Address | Memory Value | Value in Accumulator A before evaluating line of code |
| --- | --- | --- |
| 0 | LOAD IMMEDIATE 8 | XXX |
| 1 | ADD INDIRECT 7 |  |
| 2 | ADD 6 |  |
| 3 | STOP |  |
| 4 | 8 |  |
| 5 | 9 |  |
| 6 | 10 |  |
| 7 | 4 |  |
| 8 | 5 |  |
| 9 | 6 |  |
| 10 | 7 |  |
| 11 | 12 |  |
| 12 | 11 |  |

Fill in the table below with the corresponding infix.

| Infix | Postfix |
| --- | --- |
|  | 7 1 2 + * 4 5 6 * + 3 8 + * + |

Determine the Hamming-code parity bits for the following data. Assume even parity.

| 0001 | 0010 | 0011 | 0100 | 0101 | 0110 | 0111 | 1000 | 1001 | 1010 | 1011 | 1100 | 1101 | 1110 | 1111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
|  |  | 0 |  | 0 | 1 | 1 |  | 1 | 0 | 1 | 1 | 1 | 0 | 1 |

Express the following float as a decimal number. Show any intermediate steps.

Sign

1

Mantissa

| 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Exponent

| 0 | 0 | 1 | 0 | 0 | 1 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Interpret the following eight-bit binary string

| 1 | 0 | 1 | 1 | 0 | 1 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| A. As an unsigned integer |  |
| --- | --- |

| B. As a signed integer |  |
| --- | --- |

| C. As a hexadecimal number |  |
| --- | --- |

Show the state of the stack at each stage of the following postfix calculation.  Use a column of the table to represent what’s on the stack at that stage.  Start at the left and proceed to the right.

4 7 6 + 8 * 2 5 + * 9 1 + * 3 * +

|  | 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 4 | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Time 