- A 2-kg block is released from the top of a frictionless 30 incline that has a height of 1.0 m.  A spring, which an equilibrium length equal to half the length of the track and a force constant of 117.6 N/m, lies on the bottom part of the track.

- By how much is the spring compressed when the block reaches its lowest point?

- If the block were very slowly lowered by hand until it reached its equilibrium position, by how much would the spring be compressed?  (If you notice that 117.6 = 12  9.8, it might simplify your algebra.)

                - 1.0 m

                - 30

- An iron cylinder with a 1.00-cm radius and a 5.00-cm height is released from rest in a tube of water that stands 1.20 m high.  (Iron has a density 7.96  103 kg/m3.)

- Determine the buoyant force on the cylinder.

- If the cylinder is subject to a drag force Fd=38900 r2v2 (where Fd is the force in newtons, r the radius in meters and v the speed in m/s), determine the terminal speed of the cylinder.

                - 5 cm

- A 2.0-kg box is pushed 6.0 m upward along a 37o incline by a 30.0 N force (as shown in the figure below).  If the box is moving at a constant velocity of 1.5 m/s:

- How much work does the 30.0-N force do?

- How much work is done against friction?

What is the power delivered by the 30.0-N force?

                - 30 N

- 6 m

                - 37o

                - 6m

- A diameter of a horizontal pipe shrinks from 10.0 cm to 5.00 cm.  If the pressure of the water in the larger part is 1.40  105 Pa and the pressure in the smaller part is 1.20  105 Pa, what is the velocity of water in the smaller part?  How many liters of water flow past a point in the smaller part in a second?  (1 liter = 0.001 m3.)

                - 5  cm

                - 10 cm