| # PHY 106 Homework 11 | Name: ____________________ April 14, 2008 |
| --- | --- |

- The Bohr model for a Hydrogen atom has an electron (me = 9.10910-31 kg, e = -1.60210-19 C) revolving around a proton (e = +1.60210-19 C).  The radii of the allowed orbits is given by the formula

Rn = n2 ħ2 / me k e2

where n in an integer n= 1, 2, 3,  … , ħ = 1.05510-34 Js, and k=8.99 109Nm2/C2.

- Calculate the first two radii R1 and R2 for the two lowest orbits (n=1 and n=2).

- Assuming the attractive force from Coulomb’s law results in a centripetal acceleration (v2/r), determine the velocities for the lowest level (n=1).

- Assuming the energy of each of these levels is just kinetic energy plus electrical potential energy; calculate the energy of the lowest level (n=1).

- Consider the circuit drawn below.  The capacitor is uncharged when the switch is thrown connecting the 3 k resistor to the battery.

- What is the charge on the 2 mF capacitor a long time afterward?

- How much time does it take for the capacitor to acquire three-quarters as much charge?

- After the capacitor is fully charged, the switch is thrown disconnecting it from the battery and connecting it to the 2 k resistor.  How long does it take for the capacitor to lose two-thirds of its charge?

3 k

10 V

2 mF

2 k

- Calculate the current going through the 4 k and 2 k resistors in the circuit below.

2 k

4 k

3 k

4 V

5 k

6 V

- Determine the current flowing through the 2.0-kΩ resistor and the 1.5-kΩ resistor. Show work.

- There are four charges in the arrangement shown below. The two charges located at position (1, 1) and (-1, 1) are +2C, where the positions are given in centimeters.  The two charges located at (1, -1) and (-1, -1) are -2C.

  - Draw electric field lines for the charge arrangement.

  - Find the electric field E (magnitude and direction) at the origin (0, 0).

  - Find the electric potential V at the origin (0, 0).

+2C

+2C

-2C

-2C