One way to display a number is to use a seven-segment display as shown below.  A digit (0-9) is input in binary form.   The truth table for lighting the appropriate segments is shown below.

|  | W | X | Y | Z | A | b | c | d | e | f | g |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |
| 1 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 0 | 0 |
| 2 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 |
| 3 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 |
| 4 | 0 | 1 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
| 5 | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |
| 6 | 0 | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |
| 7 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 |
| 8 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 9 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 |

Enter the appropriate values for the segment d into the K-map version of the truth table.  Note that although there are 16 possible inputs, we only care about 10 of them.  You can choose these “don’t care” values as either 1 or 0, so try to choose them in a way the leads to a lot of simplification.

| Seg. e | Z | 0 | 1 | 1 | 0 |
| --- | --- | --- | --- | --- | --- |
| W | X \ Y | 0 | 0 | 1 | 1 |
| 0 | 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 | 1 |
| 1 | 0 | 1 | 1 | 1 | 1 |

Simplify the expression and write it below.

Seg. d: last column (all rows) Z’Y + Two rows (all columns) W + Four corners (first and last row, first and last column) X’Z’ + first and last row/ last two columns X’Y + Middle two rows / second column XY’Z

Build it and paste the circuit below.

Find the current through and voltage across the 2.74-kΩ and 3.16-kΩ resistors.

Using simulation. Paste your copy with meters below.

|  | Current through ( ) | Volatge across ( ) |
| --- | --- | --- |
| 2.74-kΩ |  |  |
| 3.16-kΩ |  |  |

Determine the same quantities as above by simplifying the circuit using theory/equations.

Write the “sum of products” and “product of sums” for the following truth table.

| A | B | C | Out |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 1 |
| 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 1 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 0 | 1 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 0 |
| 1 | 1 | 1 | 0 |

| Sum of products | A’B’C’ + A’B’C + A’BC’ + AB’C’ |
| --- | --- |
| Product of sums | (A+B’+C’) (A’+B+C’) (A’+B’+C) (A’+B’+C’) |