## Name:

## PHYSICS 201

LAB 7

Part 1. RC Circuit

Find the equivalent resistance of the charging circuit. Show work.

R1 & R2 parallel .848 R12 in series with R3 and R4 5.10 kilo-ohm

Find the equivalent capacitance of the charging circuit.  Show work.

C1 & C2 in series .86 micro-farad C12 in parallel with C3 2.06 micro-farad

Find the theoretical time constant of the charging circuit.  Show work.

R*C 10.5 milli-seconds

Find the equivalent resistance of the discharging circuit. Show work.

R1 & R2 in parallel .848 kilo-ohm

Find the equivalent capacitance of the discharging circuit.  Show work.

C1 & C2 in series .86 micro-farad C12 in parallel with C3 2.06 micro-farad

Find the theoretical time constant of the discharging circuit.  Show work.

R*C = 1.75 ms

Determine the simulated charging time constant.  Paste the appropriate oscilloscope screen.

10.727 ms

Determine the simulated discharging time constant.  Paste the appropriate oscilloscope screen.

1.817 ms

Part 2. Digital to Analog Conversion.

Build the following circuit.  The VDAC (voltage digital to analog converter) can be found under the MIXED button.

# A digital (eight-bit) number is entered in the top inputs.  The output is an analog voltage that corresponds to a fraction of the maximum voltage.  In particular, the fraction of the voltage is the fraction represented by the bits.  This should remind you of the mantissa of a float.  Fill in the table below.

| 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 | Voltage | % of Vmax |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  |  |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |  |
| 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |  |  |
| 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |  |  |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |  |  |
| 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 |  |  |

Part 3.

Rewire your parallel loading register (shown below) to turn it into a parallel loading shift register.    Have zeros fed in while the shifting occurs. Paste a copy of your shift register in this document.

## Part 4. Baby CRC

The circuit below is a small example of a CRC generator.  The data is fed in using the data switch D.  The clock C must go through a cycle (in particular go through a positive edge) for the shifting to occur.  When the binary string 11000001010 is fed into the CRC generator (starting with the leftmost bit), the generator has the string 1010 at the end. Build the circuit below and check this.  Change a bit in the binary string and enter it into the CRC and see if the result is different.  And so on.

| Input | CRC output |
| --- | --- |
| A. 11000001010 | 1010 |
| Change one of the bits above B. |  |
| Add four 0’s to the end Input A C. 110000010100000 |  |
| Add the output of part C to the end of Input A D. |  |
| Change two consecutive bits above E. |  |

## Part 5. Multiplication

Perform the following multiplication in detail.

|  |  |  |  |  | 1 | 0 | 1 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ´ | 1 | 0 | 1 | 1 |

|  |  |  |  |  | 1 | 1 | 0 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | ´ | 1 | 0 | 1 | 0 |