## PHYSICS 202

SPRING 07

LAB 10

## Part 1. Analog-to-Digital and Digital-to-Analog

# Give an example of a situation in which one would need an analog-to-digital converter.

## Analog-to-Digital:

# Next, give an example of a situation in which one would need a digital-to-analog converter.

## Digital-to-Analog:

## Part 2.  Analog-to-Digital Converter

# Build an analog-to-digital converter that has a two-digit output.  The output should be (1,1) if the input voltage is ¾’s or more of the reference voltage.  The output should be (1,0) if the output is between ½ and ¾’s of the reference voltage.   The output should be (0,1) if the output is between ½ and ¼ of the reference voltage.   The output should be (0,0) if the output is less than ¼ of the reference voltage.  Start with the circuit shown in the slide show, which has three outputs.  Build the circuitry that uses those three outputs as inputs and gives the two desired outputs.  Then build the circuit and paste your circuit below.  (The comparator is found under the Analog ICs buttons.)

## Part 3. Analog-to-Digital Chip

Use the ADC chip to make the circuit shown below.  Remember to use the one from the sample file.  The potentiometer (found under Basic) allows one to vary the input voltage Vin.  My screen capture below shows the potentiometer at 80%, paste a copy of your circuit showing the display for some other percentage.  Also fill in the table below with percentages and corresponding hex code.

| Percentage | Hex code |
| --- | --- |
| 0% |  |
| 20% |  |
| 40% |  |
| 60% |  |
| 80% | CC |
| 100% |  |

Part 4. A-stable multi-vibrator.

Simulate the circuit below.

Print out or paste into this document the oscilloscope display.  What happens if you change the 2-F capacitors to 4-F capacitors?

Part 5. 555 Timer as Mono-stable Multi-vibrator.

Use the 555 chip (under Mixed) to build a mono-stable multi-vibrator (one shot).  See the slide show.  See also http://www.ee.ed.ac.uk/~kap/Hard/555/node2.html#modes. Attach the trigger input to a switch.  Attach the output to an oscilloscope. Make the duration of pulse equal to 20 milliseconds.  Paste a copy of the circuit.  Paste the oscilloscope display.

R = C =

What happens if the trigger is high for longer than the duration?

Part 6. 555 Timer as A-stable Multi-vibrator.

Use the 555 chip (under Mixed) to build an a-stable multi-vibrator (oscillator).  See the slide show.  See also http://www.ee.ed.ac.uk/~kap/Hard/555/node2.html#modes.  Attach the output to an oscilloscope. Make the output have a frequency of 0.25 kiloHz and have a duty cycle of 85%.  Paste a copy of the circuit.  Paste the oscilloscope display.

Ra = Rb = C =